Generic Risk Assessment

| # Risk Assessment Title: Handling of Nanomaterials | # Date Produced: June 2023 | # Review Date: June 2026 |
| --- | --- | --- |
| # Overview/Description of Activity: Any use, handling, storage or disposal of nanomaterials as defined by the European Commission (see Nanomaterials Standard). # This risk assessment only covers the hazards associated with hazardous properties of nanomaterials and not the toxicology of the material itself. This should be covered in the associated COSHH assessment. # NB: This risk assessment assumes that a justification has already been made that nanomaterials need to be used for the work activity. | # Duration/Frequency of Activity: All work with nanomaterials regardless of duration and frequency should be risk assessed due to the uncertainty regarding their health impact. Projects can be short term, one-off activities or longer term research projects. |  |
| # Location of Activity: University of Bath facilities | # Generic or Specific Assessment: Generic assessment to be used as a basis for producing specific task-based assessments for individual projects. |  |

| # # | # Hazard(s) identified | # Who might be affected and how | # Existing controls & measures | # Severity (a) | # Likelihood (b) | # Risk Rating (a x b) | # Additional control/action required |
| --- | --- | --- | --- | --- | --- | --- | --- |
| # 1 | # Handling of particulate nanomaterials ≥1g or carrying out a machining process which could generate airborne particles | # All persons in the vicinity of the activity exposed to airborne particulate via inhalation potentially leading to respiratory disease/illness | # As a minimum, this activity must be carried out in HEPA HP14 filtered ducted containment # Minimise material becoming airborne by using in solution/matrix where practicable # Minimise number of persons who could be exposed by restricting access to work area # Minimise time of exposure | # 4 | # 2 | # 8 |  |
| # 2 | # Handling of particulate nanomaterials <1g | # All persons in the vicinity of the activity exposed to airborne particulate via inhalation potentially leading to respiratory disease/illness | # Minimise material becoming airborne by using in solution/matrix where practicable # Minimise number of persons who could be exposed by restricting access to work area # Minimise time of exposure # Appropriate containment to be justified in risk assessment to reduce the risk to as low as reasonably practicable | # 3 | # 2 | # 6 |  |
| # 3 | # Spill of particulate nanomaterial requiring clean-up which could lead to material becoming airborne | # All persons in the vicinity of the activity exposed to airborne particulate via inhalation potentially leading to respiratory disease/illness | # Use a damp cloth to minimise generating airborne particulate # If vacuum required, then it should be dedicated and HEPA filtered # Compressed air should not be used to move particulate # All waste should be considered hazardous unless it can be demonstrated that it is not | # 4 | # 2 | # 8 | # If the generation of airborne particulate cannot be avoided appropriate RPE (minimum FFP3) should be worn during clean-up |
| # 4 | # Spill of nanomaterial in solution/matrix | # Potential for splash in eye or on skin of persons in the vicinity leading to adsorption of hazardous material | # Fume Cupboard sash provides physical barrier against splashes # PPE worn as identified in the COSHH Assessment; as a minimum lab coats, safety glasses and gloves (appropriate for solvent used) # Department procedures and training # Ensure all containers are closed /lidded when not in use | # 3 | # 2 | # 6 | # Where the likelihood of a splash is possible consider the use of a face shield |
| # 5 | # Poor hygiene such as not changing gloves, washing hands when leaving lab area | # Spread of nanomaterial outside containment and potential for ingestion of nanomaterial by any person in the department leading to acute illness | # UHSE policy and procedures # Department procedures and training # No eating, drinking etc. within lab areas # Potentially contaminated gloves should not be reused | # 2 | # 3 | # 6 |  |
| # 6 | # Incorrect Waste disposal into non-hazardous waste route | # Persons collecting waste inadvertently become exposed to hazardous material, particularly as may not be contained or labelled correctly resulting in illness | # COSSH Assessment completed identifying correct waste disposal route # UHSE policy and procedures on hazardous material/waste and Nanomaterials # Department procedures, supervision and training | # 3 | # 2 | # 6 |  |
| # 7 | # Handling of High Aspect Ratio Nanomaterials (HARNs) | # All persons in the vicinity of the activity repeatedly exposed to airborne particulate via inhalation potentially leading to chronic respiratory disease/illness | # Justification must be made for use of these materials and agreed by Technical Expert/UHSE # Operations where aerosol is intentionally generated (e.g. spraying) must be carried out within a full enclosure such as a glove box # For operations where an aerosol may be produced, appropriate containment must be identified and agreed depending on factors such as amount used, frequency of operation and potential exposure time # Provide appropriate RPE in case of an accidental release of material | # 4 | # 2 | # 8 |  |
| # 8 | # Housekeeping activities such as cleaning vessels, tubes after use that contain residual nanomaterial. Cleaning activities could include washing out with water at a sink. | # Potential for material to become airborne and persons exposed to airborne particulate via inhalation potentially leading to respiratory disease/illness # Potential for splash in eye or on skin of persons in the vicinity leading to adsorption of hazardous material | # Minimise potential for material to become airborne by not allowing it to dry out # Wear appropriate PPE as defined by COSHH assessment # Ensure all areas are thoroughly cleaned of material | # 2 | # 3 | # 6 |  |
| # 9 | # Production of airborne combustible nanoparticles such as carbon-based and metal materials from processing creates a dust cloud which could result in a fire or explosion when mixed with air. | # All persons in the vicinity of the activity could be exposed to fire/explosion resulting in major injury | # All operations that produce airborne particulate will be carried out in containment which will minimise the spread of fire/contain explosion to an extent # Minimisation of amount of material # Minimise material becoming airborne by dampening where practicable # University fire procedures including presence and use of extinguishers and evacuation # DSEAR assessment to be carried out prior to any work commencing where significant quantities of combustible dust could be produced | # 5 | # 1 | # 5 |  |

| # Assessor signature: | # Print name: | Date: |
| --- | --- | --- |

| # Technical Expert signature: | # Print name: | Date: |
| --- | --- | --- |